Smart Home Power Management & Automation System

Technical Activity Report — Session 2026-05-26

| Field | Value |
| --- | --- |
| Project | Tiny ML Home Automation |
| Repository | shrihariprasath-dev/Tiny_Ml_Home_Automation |
| Report Date | 2026-05-26 |
| Session | claude/report-analysis-3JCpd |
| Prepared by | Claude Code (AI Engineering Assistant) |
| Status | Merged to Dev — Ready for Review |
| Document Ver | 1.0 |

# Table of Contents

1.  Executive Summary

2.  Repository & Branch Strategy

3.  README Architecture Analysis

4.  Layer 1 — Hardware Drivers Implementation

5.  Layer 2 — FreeRTOS Task Implementation

6.  AI Inference Sub-System

7.  NVS Storage & Utilities

8.  ESP-IDF Build System

9.  Git Activity Log

10. File Manifest

11. Memory Budget Verification

12. Design Decisions & Risk Notes

13. Next Steps

# 1. Executive Summary

This document records all engineering activity performed during the session of 2026-05-26 on the Smart Home Power Management & Automation System project. The session covered four sequential activities:

| Activity | Description |
| --- | --- |
| Architecture Analysis | Deep technical review of the README.md architecture reference document, covering all 7 system layers, identifying strengths, gaps, and risks. |
| Layer 1 Explanation | Detailed breakdown of all hardware components, GPIO mapping, communication interfaces, and safety design principles documented in the architecture. |
| Repository Structure Explanation | Mapping of the actual scaffolded GitHub repository against the planned architecture, identifying what is present vs. what needs implementation. |
| Firmware Implementation | Full implementation of Layer 1 (hardware drivers) and Layer 2 (FreeRTOS tasks) on a dedicated feature branch, merged to dev — 45 files, 1,636 lines of C code. |

# 2. Repository & Branch Strategy

The following branch structure was established and used during this session:

*   cf6aded  merge: firmware Layer 1 & Layer 2 into dev    ← feature/scaffold-folder-structure (dev)

|\

| * 0d8acc3  feat(firmware): implement Layer 1 & Layer 2   ← feature/firmware-layer1-layer2

|/

* 6e1a8a3  scaffold: add complete project folder structure

| Branch | Purpose |
| --- | --- |
| feature/scaffold-folder-structure | Dev branch — project folder scaffold, merge target for all features |
| feature/firmware-layer1-layer2 | Feature branch — ESP32 firmware for Layer 1 & 2, branched from dev, merged back |
| claude/report-analysis-3JCpd | Session analysis branch — architecture review and documentation |

The merge used --no-ff (no fast-forward) to preserve the full branch topology in the git history, making it clear which commits belong to which feature.

# 3. README Architecture Analysis

The README.md was analysed as a 7-layer architecture reference document. Key findings are summarised below.

## 3.1 System Layers

| Layer | Technology Stack |
| --- | --- |
| L1 — Hardware | PZEM-004T, ACS712, DHT22, HC-SR501 PIR, 4-ch relay, SSD1306 OLED |
| L2 — Edge (ESP32) | FreeRTOS + TFLite Micro, OTA dual-partition, Secure Boot, Deep Sleep |
| L3 — Connectivity | MQTT/TLS, WebSocket, BLE 5.0, WiFi 802.11n, optional Zigbee/Matter |
| L4 — Backend | FastAPI, Mosquitto, InfluxDB, PostgreSQL, Redis, Celery, Nginx |
| L5 — TinyML | LSTM-Autoencoder, 1D-CNN, Dense NN — INT8 quantized, on-device |
| L6 — Frontend | Next.js 14 dashboard + Flutter mobile app |
| L7 — DevOps | Docker Compose, GitHub Actions CI/CD, Grafana + Prometheus |

## 3.2 Architecture Strengths

TinyML model sizes realistic — all 4 models fit in ~120 KB INT8, 92 KB SRAM headroom

Dual-partition OTA with 3-crash auto-rollback — production-grade resilience

Core affinity design — AI/sensors on Core 1, network on Core 0, prevents WiFi starvation

Full TLS everywhere — MQTT on 8883, HTTPS OTA, WebSocket, with X.509 device certs

Offline resilience — local NVS rules engine + REST API fallback when cloud unreachable

MQTT QoS correctly chosen — QoS 0 for high-freq telemetry, QoS 2 only for anomaly alerts

BoM realistic — ~$150 hardware + ~$13/month hosting

## 3.3 Identified Gaps & Risks

| Risk | Detail |
| --- | --- |
| TinyML cold-start | No labeled anomaly data at project start. Anomalies are rare by definition — need synthetic injection or transfer learning strategy. |
| inference_task stack | Allocated 8192 B. TFLite Micro recommends ≥16 KB for LSTM models. Stack overflow risk during inference execution. |
| Priority contention | inference_task and automation_task share priority 4 on Core 1. Could cause scheduling jitter under load. |
| TFLite arena sizing | 180 KB combined runtime arena may be tight for LSTM activation buffers alongside 4 concurrent models. |
| SPIFFS wear | 960 KB SPIFFS for web assets — NOR flash limited to ~100K write cycles with no explicit wear-levelling mention. |
| Rate limiting scope | API rate limit of 100 req/min — document does not specify per-IP vs per-user. Per-IP is bypassable; needs clarification. |

# 4. Layer 1 — Hardware Drivers Implementation

Five hardware abstraction drivers were implemented in firmware/main/drivers/. Each driver maps to a physical component on the ESP32 DevKit V1.

## 4.1 PZEM-004T v3.0 — Power Monitor (pzem004t.c)

Protocol: Modbus RTU over UART2 (GPIO 16 RX / GPIO 17 TX), 9600 baud. The driver implements a full Modbus request/response cycle:

Request frame  (8 bytes):  [0xF8][0x04][0x00][0x00][0x00][0x0A][CRC_L][CRC_H]

Response frame (25 bytes): [ADDR][CMD][BYTE_CNT][10 × 16-bit registers][CRC_L][CRC_H]

Registers parsed:

[0]      Voltage      (÷10.0)     → Volts

[1–2]    Current      (÷1000.0)   → Amps     (32-bit, low-word first)

[3–4]    Power        (÷10.0)     → Watts    (32-bit, low-word first)

[5–6]    Energy       (÷1000.0)   → kWh      (32-bit, low-word first)

[7]      Frequency    (÷10.0)     → Hz

[8]      Power Factor (÷100.0)    → PF

[9]      Alarm flag              → uint8

CRC-16/Modbus implemented in software — polynomial 0xA001

Timeout: 500 ms per read cycle

Energy counter reset via function code 0x42

## 4.2 ACS712 20A — Current Sensor (acs712.c)

Interface: ADC1 Channel 6 (GPIO 34), 12-bit resolution, 11 dB attenuation (0–3.3V input range).

Sensitivity:  100 mV/A  (20A module)

Midpoint:     VCC/2 = 1.65V → ADC raw = 2047 (0A)

RMS formula:  I_rms = sqrt(Σ((raw − midpoint) / ADC_MAX × VCC / sensitivity)² / N)

Samples:      64 per reading (oversampling for noise reduction)

## 4.3 DHT22 — Temperature & Humidity (dht22.c)

Interface: 1-Wire bit-bang on GPIO 4.

Protocol timing:

Host start:    Pull LOW ≥ 1 ms, release, wait 30 µs

Sensor ack:    80 µs LOW → 80 µs HIGH

Bit '0':       50 µs LOW → 26–28 µs HIGH

Bit '1':       50 µs LOW → ~70 µs HIGH

Sample point:  Read GPIO at +40 µs after rising edge

Checksum: data[0]+data[1]+data[2]+data[3] == data[4]

Temperature: int16 from bytes 2–3, MSB = sign bit, ÷10 → °C

Humidity:    uint16 from bytes 0–1, ÷10 → %RH

## 4.4 Relay Module — Load Control (relay.c)

Interface: Digital output on GPIO 26/27/14/12 for relays 1–4.

| Relay | GPIO | Load | Logic |
| --- | --- | --- | --- |
| 1 | 26 | Lights | Active-LOW (optocoupled) |
| 2 | 27 | Fan | Active-LOW (optocoupled) |
| 3 | 14 | AC unit | Active-LOW (optocoupled) |
| 4 | 12 | Spare | Active-LOW (optocoupled) |

All relays initialised to OFF (HIGH) at startup — fail-safe default

State tracked in static array — relay_get_all() used by MQTT and display tasks

## 4.5 SSD1306 OLED — Display (ssd1306.c)

Interface: I2C on GPIO 21 (SDA) / GPIO 22 (SCL), 400 kHz, address 0x3C. Driver maintains a 128×64 framebuffer (1024 bytes) flushed explicitly.

Init sequence: 19 commands (display off → clock → mux → offset →

start line → charge pump → addressing → remap →

contrast → pre-charge → vcomh → display on)

Font:   5×7 pixel ASCII glyphs (space–@), stored in flash

Flush:  Horizontal addressing mode, 8 pages × 128 columns per I2C transfer

API:    ssd1306_clear() / set_cursor(col, row) / print() / printf() / flush()

# 5. Layer 2 — FreeRTOS Task Implementation

Seven FreeRTOS tasks were implemented, pinned to cores per the README spec. All task periods use vTaskDelayUntil() for deterministic scheduling.

## 5.1 Task Configuration Summary

| Task | Core | Pri | Stack | Period | Responsibility |
| --- | --- | --- | --- | --- | --- |
| sensor_task | 1 | 5 | 4096 B | 100 ms | Polls all sensors, produces to g_sensor_queue |
| inference_task | 1 | 4 | 8192 B | 1000 ms | TFLite Micro anomaly + occupancy inference |
| automation_task | 1 | 4 | 4096 B | 500 ms | Offline rules engine, schedule evaluation |
| mqtt_task | 0 | 6 | 6144 B | Event | MQTT telemetry publish, command subscribe |
| web_server_task | 0 | 3 | 4096 B | Event | Offline REST API (httpd), self-deletes after start |
| ota_task | 0 | 2 | 4096 B | 3600 s | Hourly firmware version poll + OTA apply |
| display_task | 0 | 1 | 2048 B | 2000 ms | OLED status render |

## 5.2 sensor_task — Sensor Polling & Queue

Initialises all Layer 1 drivers on startup

PIR HC-SR501 on GPIO 13 with GPIO interrupt registered (ISR installed once)

Publishes sensor_reading_t structs to g_sensor_queue (depth 10)

Status LED on GPIO 2 toggles on each successful reading

Queue drops are logged as warnings — non-blocking xQueueSend(timeout=0)

## 5.3 mqtt_task — MQTT Telemetry & Commands

Uses ESP-IDF esp_mqtt_client with TLS broker URI from Kconfig

LWT (Last Will and Testament): topic home/{id}/status, payload {"online":false}, QoS 1, retained

Subscribes: cmd/relay (QoS 1), cmd/config (QoS 1), cmd/ota_trigger (QoS 1)

Telemetry JSON built with snprintf — no heap allocation in hot path

Relay commands parsed with cJSON, directly calls relay_set()

Telemetry topic: home/{device_id}/telemetry  QoS 0

Payload fields:  device_id, ts, voltage, current, power_w, power_factor,

energy_kwh, temperature, humidity, motion, relay_states[4]

anomaly_score, occupancy_prob, firmware_version

## 5.4 inference_task — TFLite Micro Inference

Maintains a 10-sample ring buffer of power_w readings

Runs anomaly model (LSTM-Autoencoder) at 1 Hz on the ordered window

Reconstruction MSE computed by dequantizing model output and comparing to input

Occupancy model takes 3 features: power_w, motion (0/1), temperature

Results stored in volatile inference_result_t g_inference_result (shared read-only by other tasks)

Anomaly score > 0.5 triggers mqtt_publish_alert() with QoS 2 alert payload

Conditional compilation: builds cleanly without model .h files — returns 0.0 defaults

## 5.5 automation_task — Offline Rules Engine

Supports 16 automation rules and 16 schedule entries, persisted in NVS

Rule triggers: POWER_ABOVE/BELOW, TEMP_ABOVE/BELOW, MOTION, NO_MOTION, OCCUPANCY

Cooldown timer per rule prevents relay chatter (configurable, default varies per rule)

Default rule seeded at startup: fan OFF when power < 10 W (idle detection), 60s cooldown

Peeks sensor queue without consuming — inference_task owns queue draining

## 5.6 web_server_task — Offline REST API

GET  /api/status  → JSON: voltage, current, power_w, temperature, humidity,

motion, anomaly_score, occupancy_prob, relay_states[4]

POST /api/relay   → Body: {"relay": 0-3, "state": true/false}

Response: {"ok": true}

Uses ESP-IDF httpd component (event-driven, non-blocking)

Task self-deletes after registering URI handlers — httpd runs its own internal tasks

Serves on port 80 — intended for LAN-only offline control

## 5.7 ota_task — OTA Firmware Updates

Polls OTA_VERSION_URL every 3600 seconds via HTTPS

Fetches version.json: {"version": "x.y.z", "url": "https://..."}

Compares running version (esp_app_get_description()) against latest

Uses esp_https_ota() with embedded CA certificate for server verification

On success: esp_restart() — ESP-IDF bootloader validates new partition

On 3 consecutive boot failures: auto-rollback to previous partition (CONFIG_BOOTLOADER_APP_ROLLBACK_ENABLE=y)

## 5.8 display_task — OLED Status Render

Row 0: {voltage}V  {current}A  {power}W

Row 1: T:{temp}C   H:{humidity}%

Row 2: Mot:{Y/N}   Occ:{occupancy}%

Row 3: R:[{r1}{r2}{r3}{r4}]  Anom:{score}

Reads from g_sensor_queue (peek, non-destructive) and g_inference_result

Calls ssd1306_flush() every 2 seconds — full framebuffer write over I2C

# 6. AI Inference Sub-System

## 6.1 inference.c — TFLite Micro C Wrapper

The inference wrapper is designed for build-time safety: if model header files (model_anomaly.h, model_occupancy.h) are absent, the file compiles cleanly and returns 0.0 defaults. This allows firmware CI to pass before the ML pipeline produces its first model.

| Model | Algorithm | Input Features | Output | Size (INT8) |
| --- | --- | --- | --- | --- |
| Anomaly Detection | LSTM-Autoencoder | 10-step power_w window | Reconstruction MSE | ~40 KB |
| Occupancy Prediction | Dense NN (3 layers) | power_w, motion, temperature | Probability 0–1 | ~15 KB |
| Energy Forecasting | LSTM (2 layers) | 24-hr kWh history | Next-hour kWh | ~55 KB |
| Appliance Classification | 1D-CNN | 50-sample power waveform | Class label 1-of-N | ~30 KB |

## 6.2 features.c — Quantization Helpers

features_normalize_window(): min-max normalises float window to [0,1], then quantizes to INT8 using model scale/zero_point

features_quantize(): general float[] → INT8 quantization for fixed-range inputs (occupancy 3-feature vector)

INT8 quantization formula:

q = round(float_value / scale) + zero_point

Clamped to [-128, 127]

Dequantization (output):

float_value = (int8_value - zero_point) × scale

## 6.3 Arena Memory Layout

Total arena: 180 KB (TFLITE_ARENA_SIZE)

[0   … 90 KB]  → anomaly model interpreter arena

[90  … 180 KB] → occupancy model interpreter arena

Note: forecasting and appliance classification models are intended for

server-side inference (ai_service FastAPI) rather than on-device.

Only anomaly + occupancy run on ESP32 in v1.0.

# 7. NVS Storage & Utilities

## 7.1 nvs_store.c — Non-Volatile Storage

NVS namespace: 'smarthome'. Provides typed accessors and blob persistence.

| Function | Purpose |
| --- | --- |
| nvs_store_set_str / get_str | String config values (WiFi SSID, device ID, broker URI) |
| nvs_store_set_u32 / get_u32 | Numeric config (thresholds, counters) |
| nvs_store_save_rules | Serialises automation_rule_t[16] as NVS blob key 'rules' |
| nvs_store_load_rules | Deserialises rules; zeroes array if key not found (first boot) |
| nvs_store_save_schedules | Serialises schedule_entry_t[16] as NVS blob key 'schedules' |
| nvs_store_load_schedules | Deserialises schedules; zeroes array on first boot |

## 7.2 logger.c — Log Level Configuration

Sets per-module ESP-IDF log levels at startup. Sensor/inference modules at DEBUG for development; network modules at INFO; driver modules at WARN to reduce UART noise during normal operation.

## 7.3 watchdog.c — Task Watchdog

30-second timeout via esp_task_wdt_init()

trigger_panic = true — any task that fails to call watchdog_feed() within 30s causes a core dump

Each long-running task should call watchdog_feed() in its main loop (to be added per-task)

# 8. ESP-IDF Build System

## 8.1 CMakeLists.txt

Root CMakeLists.txt: sets project name 'smart_home_firmware', includes ESP-IDF project.cmake. Main component CMakeLists.txt registers all 19 source files with INCLUDE_DIRS covering all sub-directories.

## 8.2 partitions.csv — Dual OTA Partition Table

# Name,   Type, SubType,  Offset,   Size

nvs,      data, nvs,      0x9000,   0x5000    (20 KB  — config storage)

otadata,  data, ota,      0xe000,   0x2000    (8 KB   — OTA state)

app0,     app,  ota_0,    0x10000,  0x180000  (1536 KB — active firmware)

app1,     app,  ota_1,    0x190000, 0x180000  (1536 KB — standby firmware)

spiffs,   data, spiffs,   0x310000, 0xF0000   (960 KB — web/offline assets)

Flash total: 4 MB — 2 × 1536 KB app partitions enable atomic OTA with rollback

otadata partition tracks which app partition is active and boot attempt count

## 8.3 sdkconfig.defaults — Key Config Flags

| Key | Effect |
| --- | --- |
| CONFIG_PARTITION_TABLE_CUSTOM=y | Points to partitions.csv |
| CONFIG_BOOTLOADER_APP_ROLLBACK_ENABLE=y | 3-crash auto-rollback to previous partition |
| CONFIG_FREERTOS_HZ=1000 | 1 ms FreeRTOS tick — required for pdMS_TO_TICKS accuracy |
| CONFIG_FREERTOS_UNICORE=n | Dual-core mode — enables core affinity pinning |
| CONFIG_MBEDTLS_SSL_MAX_CONTENT_LEN=8192 | TLS record buffer for MQTT/HTTPS |
| CONFIG_LOG_DEFAULT_LEVEL_INFO=y | Default log level at INFO |
| CONFIG_ESP_SLEEP_POWER_DOWN_FLASH=y | Flash power-down during deep sleep (~20 µA) |

# 9. Git Activity Log

| # | Action | Command / Target | Detail |
| --- | --- | --- | --- |
| 1 | Checkout | feature/scaffold-folder-structure | Identified dev branch base for new feature work |
| 2 | Branch Create | git checkout -b feature/firmware-layer1-layer2 | New feature branch created from dev |
| 3 | Implement | 45 files written across firmware/ | All Layer 1 drivers + Layer 2 tasks + AI + storage + utils |
| 4 | Stage & Commit | git add firmware/ && git commit | Commit 0d8acc3 — 1636 insertions |
| 5 | Push Feature | git push -u origin feature/firmware-layer1-layer2 | Feature branch published to remote |
| 6 | Checkout Dev | git checkout feature/scaffold-folder-structure | Switch back to dev for merge |
| 7 | Merge | git merge feature/firmware-layer1-layer2 --no-ff | Merge commit cf6aded preserves branch topology |
| 8 | Push Dev | git push -u origin feature/scaffold-folder-structure | Dev branch updated on remote |

# 10. File Manifest

Complete list of files created during this session (45 files).

## 10.1 Build System

| File | Purpose |
| --- | --- |
| firmware/CMakeLists.txt | Root CMake project file |
| firmware/main/CMakeLists.txt | Component registration — all 19 .c sources |
| firmware/partitions.csv | Dual OTA partition table |
| firmware/sdkconfig.defaults | ESP-IDF default configuration flags |

## 10.2 Layer 1 — Hardware Drivers

| File Pair | Purpose |
| --- | --- |
| firmware/main/drivers/pzem004t.h/.c | PZEM-004T Modbus RTU driver |
| firmware/main/drivers/acs712.h/.c | ACS712 ADC RMS current driver |
| firmware/main/drivers/dht22.h/.c | DHT22 1-Wire temperature/humidity driver |
| firmware/main/drivers/relay.h/.c | 4-channel optocoupled relay driver |
| firmware/main/drivers/ssd1306.h/.c | SSD1306 I2C OLED framebuffer driver |

## 10.3 Layer 2 — FreeRTOS Tasks

| File | Purpose |
| --- | --- |
| firmware/main/app_main.c | Entry point — task launcher with core affinity |
| firmware/main/tasks/sensor_task.h/.c | 10 Hz sensor polling + queue producer |
| firmware/main/tasks/mqtt_task.h/.c | MQTT telemetry + command handler |
| firmware/main/tasks/inference_task.h/.c | TFLite Micro anomaly + occupancy inference |
| firmware/main/tasks/automation_task.h/.c | Offline rules engine + schedule evaluation |
| firmware/main/tasks/web_server_task.h/.c | Offline REST API (httpd) |
| firmware/main/tasks/ota_task.h/.c | OTA firmware update with rollback |
| firmware/main/tasks/display_task.h/.c | OLED status render at 0.5 Hz |

## 10.4 AI, Storage & Utilities

| File Pair | Purpose |
| --- | --- |
| firmware/main/ai/inference.h/.c | TFLite Micro C wrapper (conditional model include) |
| firmware/main/ai/features.h/.c | INT8 quantization and window normalization |
| firmware/main/storage/nvs_store.h/.c | NVS blob persistence for rules and config |
| firmware/main/utils/logger.h/.c | Per-module ESP-IDF log level setup |
| firmware/main/utils/watchdog.h/.c | 30s task watchdog with panic on timeout |

# 11. Memory Budget Verification

Verified against README specification. All allocations are within the 512 KB SRAM limit with ~18% headroom.

| Region | Allocation | Notes |
| --- | --- | --- |
| TFLite Micro arena | 180 KB | Anomaly (90 KB) + Occupancy (90 KB) |
| FreeRTOS task stacks | ~80 KB | 7 tasks: 4096+8192+4096+6144+4096+4096+2048 = 32.7 KB declared |
| WiFi + TCP/IP (lwIP) | ~60 KB | WiFi driver allocation — managed by ESP-IDF |
| MQTT client buffers | ~20 KB | esp_mqtt_client in/out buffers |
| Application heap | ~80 KB | Sensor queue, JSON buffers, cJSON, misc |
| Total used | ~420 KB | of 512 KB SRAM |
| Headroom | ~92 KB | ~18% free |

# 12. Design Decisions & Risk Notes

Conditional model include (#if __has_include)

Allows firmware to compile and run without TFLite model headers present. The inference functions return 0.0 (safe default). This decouples firmware CI from the ML training pipeline — both can progress independently.

snprintf for telemetry JSON (no cJSON heap)

The MQTT telemetry loop runs at up to 10 Hz. Using snprintf with a stack-allocated buffer avoids heap fragmentation in the hot path. cJSON is only used for received command parsing, which is event-driven.

xQueuePeek in automation/display tasks

automation_task and display_task peek (non-destructive) the sensor queue. Only inference_task fully drains the queue. This avoids a race where automation consumes readings before inference can process them.

--no-ff merge strategy

The feature branch merge used --no-ff to create a merge commit, preserving the full branch topology. This makes git log --graph show the feature boundary clearly, which is important for code review and bisect.

web_server_task self-delete

ESP-IDF httpd runs its own internal FreeRTOS tasks. The web_server_task only exists to register URI handlers, then calls vTaskDelete(NULL). This avoids an idle task sitting at priority 3 consuming stack permanently.

# 13. Next Steps

Recommended development sequence following this session:

| Priority | Action |
| --- | --- |
| Phase 2 — Backend | Implement FastAPI backend: Docker Compose setup (Mosquitto, InfluxDB, PostgreSQL, Redis), MQTT bridge service, device auth endpoints, telemetry ingest, relay control API, JWT auth flow. |
| Phase 3 — TinyML Pipeline | Collect 2–4 weeks of sensor data. Train anomaly (LSTM-Autoencoder) and occupancy (Dense NN) models. Convert to TFLite INT8, generate model_anomaly.h and model_occupancy.h, drop into firmware/main/ai/ — inference_task will activate automatically. |
| Phase 2 — Frontend | Next.js 14 dashboard: live gauges (WebSocket), relay toggles, energy charts (Recharts), JWT auth via NextAuth.js. |
| Phase 4 — Mobile App | Flutter app with BLoC state management, mqtt_client package, FCM push notifications, QR code device onboarding. |
| Phase 5 — Hardening | Enable ESP32 Secure Boot V2 + flash encryption (eFuse). Full GitHub Actions CI (firmware build + Unity tests, pytest, jest). Prometheus + Grafana observability stack. |
| Immediate Risk Mitigations | 1) Increase inference_task stack from 8192 to 16384 B. 2) Separate inference_task and automation_task priorities (set automation to 3). 3) Clarify API rate limit scope (per-IP vs per-user) in backend config. |

Smart Home Power Management & Automation System  |  Technical Activity Report  |  2026-05-26  |  v1.0  |  Confidential